# CRANBERRY ALLSPICE #EU26363F

Safety Data Sheet

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

according to the REACH Regulation (EC) 1907/2006 amended by Regulation (EU) 2020/878 Issue date: 8/21/2020 Revision date: 01/08/2026 Version: 3.0

Product form	: Mixture

Trade name	: CRANBERRY ALLSPICE #EU26363F

UFI	: UE20-V051-R00G-03HM

Product code	: EU26363F

Type of product	: Perfumes, fragrances

Product group	: Trade product

1.2. Relevant identified uses of the substance or mixture and uses advised against

## Relevant identified uses

Main use category	: Industrial use,Professional use

Industrial/Professional use spec	: For professional use only Industrial

Use of the substance/mixture	: Perfumes, fragrances

Function or use category	: Odour agents

## Uses advised against

## Do not use for purposes other than those specified in this safety data sheet and in the manufacturer’s technical documentation. For professional and industrial use only. Do not use for consumer applications, food, feed, cosmetic or medical purposes.

1.3. Details of the supplier of the safety data sheet

UAB „MANFIRA“
Company code: 302812448
Eišiškių pl. 127, LT-02184 Vilnius, Lithuania
Tel.: +370 633 11451
Email: info@craftshop.lt 
Website: www.craftshop.lt
Alternative contact email: info@krevo.eu 
Alternative website: www.krevo.eu

Email of the person responsible for the safety data sheet: info@uficode.eu 
Website: www.uficode.eu

1.4. Emergency telephone number

In case of emergency, contact the national poison centre, emergency medical service or local healthcare provider.

European emergency number: 112

National poison centre / emergency medical information numbers:

Austria: +43 1 406 43 43
Belgium: +32 70 245 245
Bulgaria: 112
Croatia: +385 1 2348 342
Cyprus: 1401
Czech Republic: +420 224 919 293; +420 224 915 402
Denmark: +45 82 12 12 12
Estonia: 16662
Finland: +358 9 471 977; +358 9 4711
France: +33 1 45 42 59 59
Germany: 112 / national poison information centre depending on federal state
Greece: +30 210 779 3777
Hungary: +36 80 201 199
Ireland: +353 1 809 2166; +353 1 809 2566
Italy: 112 / national poison centre depending on region
Latvia: +371 67042473
Lithuania: +370 5 236 2052; +370 687 53378
Luxembourg: +352 8002 5500
Malta: 112
Netherlands: +31 88 755 8000
Poland: 112 / local toxicological information centre
Portugal: +351 800 250 250
Romania: +40 21 318 3606
Slovakia: +421 2 5477 4166
Slovenia: +386 1 522 1293
Spain: +34 91 562 04 20
Sweden: 112 / Swedish Poisons Information Centre
For the current list of national appointed bodies / poison centres in the EU, see:
https://poisoncentres.echa.europa.eu/appointed-bodies

Notes:
Availability, scope and language of service may differ by country. Some numbers may be intended for healthcare professionals only. Where no specific public poison centre number is available, contact the European emergency number 112.

| SECTION 2: Hazards identification |  |
| --- | --- |
| 2.1. Classification of the substance or mixture |  |
| Classification according to Regulation (EC) No. 1272/2008 [CLP] |  |
| Acute toxicity (dermal), Category 4 | H312 |
| Skin corrosion/irritation, Category 2 | H315 |
| Serious eye damage/eye irritation, Category 2 | H319 |
| Skin sensitisation, Category 1 | H317 |
| Carcinogenicity, Category 1B | H350 |
| Aspiration hazard, Category 1 | H304 |
| Hazardous to the aquatic environment – Chronic Hazard, Category 3 Full text of H- and EUH-statements: see section 16 | H412 |
| Adverse physicochemical, human health and environmental effects |  |

Toxic to aquatic life with long lasting effects. May cause cancer. Harmful in contact with skin. Harmful if inhaled. Harmful if swallowed. Causes skin irritation. Causes serious eye irritation. May be fatal if swallowed and enters airways. Harmful to aquatic life with long lasting effects. May cause an allergic skin reaction.

2.2. Label elements

## Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP)	:

GHS07	GHS08

Signal word (CLP)	: Danger

Contains	: Cinnamic aldehyde; Eugenol; d-Limonene; COUMARIN; Nutmeg Oil

Hazard statements (CLP)	: H302+H312+H332 - Harmful if swallowed, in contact with skin or if inhaled.

H304 - May be fatal if swallowed and enters airways. H315 - Causes skin irritation.

H317 - May cause an allergic skin reaction. H319 - Causes serious eye irritation.

H350 - May cause cancer.

H412 - Harmful to aquatic life with long lasting effects.

Precautionary statements (CLP)	: P201 - Obtain special instructions before use.

P202 - Do not handle until all safety precautions have been read and understood. P261 - Avoid breathing dust/fume/gas/mist/vapours/spray.

P264 - Wash hands, forearms and face thoroughly after handling. P270 - Do not eat, drink or smoke when using this product.

P271 - Use only outdoors or in a well-ventilated area.

Extra phrases	: For professional users only.

# 2.3. Other hazards

Contains no PBT/vPvB substances ≥ 0.1% assessed in accordance with REACH Annex XIII

The mixture does not contain substance(s) included in the list established in accordance with Article 59(1) of REACH for having endocrine disrupting properties, or is not identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0,1 %

SECTION 3: Composition/information on ingredients

# Substances

Not applicable

# Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Cinnamic aldehyde | CAS-No.: 104-55-2 EC-No.: 203-213-9 REACH-no: 01-2119935242- 45 | 10.8 – 21.645 | Acute Tox. 4 (Dermal), H312 Skin Irrit. 2, H315 Eye Irrit. 2, H319 Skin Sens. 1, H317 Aquatic Chronic 3, H412 |
| Eugenol | CAS-No.: 97-53-0 EC-No.: 202-589-1 REACH-no: 01-2119971802- 33 | 5.4 – 10.855 | Acute Tox. 4 (Oral), H302 Eye Irrit. 2, H319 Skin Sens. 1B, H317 |
| d-Limonene substance with national workplace exposure limit(s) (DE, ES, FI, SI, NO, CH) | CAS-No.: 5989-27-5 EC-No.: 227-813-5 EC Index-No.: 601-096-00-2 REACH-no: 01-2119493353- 35 | 5.4 – 10.855 | Flam. Liq. 3, H226 Skin Irrit. 2, H315 Skin Sens. 1B, H317 Asp. Tox. 1, H304 Aquatic Acute 1, H400 (M=1) Aquatic Chronic 3, H412 |
| COUMARIN | CAS-No.: 91-64-5 EC-No.: 202-086-7 REACH-no: 01-2119943756- 26 | 5.1 – 10.14 | Acute Tox. 4 (Oral), H302 Skin Sens. 1B, H317 |
| Methyl ester of rosin (partially hydrogenated) | CAS-No.: 8050-15-5 EC-No.: 232-476-2 | 3 – 6.045 | Aquatic Chronic 3, H412 |
| Bis(2-ethylhexyl) adipate substance with national workplace exposure limit(s) (PL) | CAS-No.: 103-23-1 EC-No.: 203-090-1 REACH-no: 01-2119439699- 19 | 1 – 1.95 | Not classified |
| Phenylpropyl alcohol | CAS-No.: 122-97-4 EC-No.: 204-587-6 | 0.7 – 1.365 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 |
| Nutmeg Oil | CAS-No.: 8008-45-5 EC-No.: 282-013-3;616-921-1 | 0.4 – 0.715 | Flam. Liq. 3, H226 Skin Sens. 1, H317 Muta. 2, H341 Carc. 1B, H350 Repr. 2, H361 Asp. Tox. 1, H304 Aquatic Chronic 2, H411 |
| Amyl salicylate | CAS-No.: 2050-08-0 EC-No.: 218-080-2 REACH-no: 01-2119969444- 27 | 0.2 – 0.455 | Acute Tox. 4 (Oral), H302 Aquatic Chronic 1, H410 (M=1) |

Full text of H- and EUH-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

First-aid measures general: If exposed or concerned, get medical advice/attention. Call a poison centre or doctor if you feel unwell. Show this safety data sheet or product label to medical personnel.

First-aid measures after inhalation: Remove person to fresh air and keep comfortable for breathing. If symptoms occur or persist, get medical advice/attention. Call a poison centre or doctor if you feel unwell.

First-aid measures after skin contact: Wash skin with plenty of water. Take off contaminated clothing and wash it before reuse. If skin irritation or rash occurs: Get medical advice/attention. In case of exposure or concern, get medical advice/attention.

First-aid measures after eye contact: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention.

First-aid measures after ingestion: Rinse mouth. Do not induce vomiting. Call a poison centre or doctor immediately. If vomiting occurs, keep head low to prevent aspiration into the lungs.

4.2. Most important symptoms and effects, both acute and delayed

Symptoms/effects after inhalation: Harmful if inhaled. Vapours, mist or spray may cause respiratory tract irritation, headache, dizziness or discomfort.

Symptoms/effects after skin contact: Harmful in contact with skin. Causes skin irritation. May cause an allergic skin reaction. Symptoms may include redness, itching, rash, dryness or irritation.

Symptoms/effects after eye contact: Causes serious eye irritation. Symptoms may include redness, pain, tearing and blurred vision.

Symptoms/effects after ingestion: Harmful if swallowed. May be fatal if swallowed and enters airways. Ingestion may cause gastrointestinal irritation, nausea, vomiting or abdominal discomfort. Aspiration into the lungs may cause chemical pneumonitis.

Delayed symptoms/effects: May cause cancer. Effects may be delayed

4.3. Indication of any immediate medical attention and special treatment needed

Treat symptomatically. Immediate medical attention is required in case of ingestion due to aspiration hazard. In case of exposure or concern, seek medical advice. Do not induce vomiting unless instructed by medical personnel.

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media	: Sand. Water spray. Dry powder. Foam. Carbon dioxide.

Unsuitable extinguishing media	: Do not use a heavy water stream.

5.2. Special hazards arising from the substance or mixture

Hazardous decomposition products in case of fire	: Toxic fumes may be released.

5.3. Advice for firefighters

Firefighting instructions	: Use water spray or fog for cooling exposed containers. Exercise caution when fighting any chemical fire. Prevent fire fighting water from entering the environment.

Protection during firefighting	: Do not enter fire area without proper protective equipment, including respiratory protection.

Do not attempt to take action without suitable protective equipment. Self-contained breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

## For non-emergency personnel

Emergency procedures	: Ventilate spillage area. Evacuate unnecessary personnel. Only qualified personnel equipped with suitable protective equipment may intervene. Avoid breathing dust/fume/gas/mist/vapours/spray.

## For emergency responders

Protective equipment	: Do not attempt to take action without suitable protective equipment. Equip cleanup crew with proper protection. For further information refer to section 8: "Exposure controls/personal protection".

Emergency procedures	: Ventilate area.

6.2. Environmental precautions

Avoid release to the environment. Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters. Notify authorities if product enters sewers or public waters.

6.3. Methods and material for containment and cleaning up

For containment	: Collect spillage.

Methods for cleaning up	: Take up liquid spill into absorbent material. Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Collect spillage. Store away from other materials. Notify authorities if product enters sewers or public waters.

Other information	: Dispose of materials or solid residues at an authorized site.

6.4. Reference to other sections

See Section 8. Exposure controls and personal protection. For further information refer to section 13.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

Precautions for safe handling	: Ensure good ventilation of the work station. Wash hands and other exposed areas with mild

soap and water before eating, drinking or smoking and when leaving work. Provide good ventilation in process area to prevent formation of vapour. Obtain special instructions before use. Do not handle until all safety precautions have been read and understood. Take all necessary technical measures to avoid or minimize the release of the product on the workplace. Limit quantities of product at the minimum necessary for handling and limit the number of exposed workers. Provide local exhaust or general room ventilation. Wear personal protective equipment. Floors, walls and other surfaces in the hazard area must be cleaned regularly. Do not get in eyes, on skin, or on clothing. Avoid breathing dust/fume/gas/mist/vapours/spray.

Hygiene measures	: Wash hands thoroughly after handling. Separate working clothes from town clothes.

Launder separately. Wash contaminated clothing before reuse. Contaminated work clothing should not be allowed out of the workplace. Do not eat, drink or smoke when using this product. Always wash hands after handling the product.

7.2. Conditions for safe storage, including any incompatibilities

Storage conditions: Store in a tightly closed container in a cool, dry and well-ventilated place. Keep away from heat, hot surfaces and direct sunlight.

Incompatible materials: Store away from incompatible materials. See section 10.

Special rules on packaging: Keep only in the original or suitable container. Do not store in corrodable metal containers.

Joint storage: Store in accordance with applicable national regulations and good storage practice for chemical products. Follow any local requirements for segregation from incompatible substances.

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

## 8.1.1 National occupational exposure and biological limit values

| d-Limonene (5989-27-5) |  |
| --- | --- |
| Finland - Occupational Exposure Limits |  |
| HTP (OEL TWA) [1] | 140 mg/m³ |
| HTP (OEL TWA) [2] | 25 ppm |
| HTP (OEL STEL) | 280 mg/m³ |
| HTP (OEL STEL) [ppm] | 50 ppm |
| Germany - Occupational Exposure Limits (TRGS 900) |  |
| AGW (OEL TWA) [1] | 28 mg/m³ (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| AGW (OEL TWA) [2] | 5 ppm (the risk of damage to the embryo or fetus can be excluded when AGW and BGW values are observed) |
| Chemical category | Skin notation, Skin sensitization |
| Slovenia - Occupational Exposure Limits |  |
| OEL TWA | 28 mg/m³ |

| d-Limonene (5989-27-5) |  |
| --- | --- |
| OEL TWA [ppm] | 5 ppm |
| OEL STEL | 112 mg/m³ |
| OEL STEL [ppm] | 20 ppm |
| OEL chemical category | Potential for cutaneous absorption |
| Spain - Occupational Exposure Limits |  |
| VLA-ED (OEL TWA) [1] | 168 mg/m³ |
| VLA-ED (OEL TWA) [2] | 30 ppm |
| OEL chemical category | Sensitizer, skin - potential for cutaneous absorption |
| Norway - Occupational Exposure Limits |  |
| Grenseverdi (OEL TWA) [1] | 140 mg/m³ |
| Grenseverdi (OEL TWA) [2] | 25 ppm |
| Korttidsverdi (OEL STEL) | 175 mg/m³ (value calculated) |
| Korttidsverdi (OEL STEL) [ppm] | 37.5 ppm (value calculated) |
| OEL chemical category | Allergenic substance |
| Switzerland - Occupational Exposure Limits |  |
| MAK (OEL TWA) [1] | 40 mg/m³ |
| MAK (OEL TWA) [2] | 7 ppm |
| KZGW (OEL STEL) | 80 mg/m³ |
| KZGW (OEL STEL) [ppm] | 14 ppm |
| OEL chemical category | Sensitizer |
| Bis(2-ethylhexyl) adipate (103-23-1) |  |
| Poland - Occupational Exposure Limits |  |
| NDS (OEL TWA) | 400 mg/m³ |

    - Recommended monitoring procedures

No additional information available

## Air contaminants formed

No additional information available

## DNEL and PNEC

No additional information available

## Control banding

No additional information available

# Exposure controls

## Appropriate engineering controls Appropriate engineering controls:

Ensure good ventilation of the work station.

## Personal protection equipment Personal protective equipment:

Avoid all unnecessary exposure.

## Personal protective equipment symbol(s):

      - Eye and face protection Eye protection:

Chemical goggles or safety glasses. Safety glasses

## Skin protection Skin and body protection:

Wear suitable protective clothing

## Hand protection:

Wear protective gloves.

## Respiratory protection Respiratory protection:

Wear appropriate mask. [In case of inadequate ventilation] wear respiratory protection.

## Thermal hazards

No additional information available

## Environmental exposure controls Environmental exposure controls:

Avoid release to the environment.

## Other information:

Do not eat, drink or smoke during use.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

Physical state	: Liquid

Colour	: light yellow. amber. Conforms to standard.

Odour	: characteristic.

Odour threshold	: Not available

Melting point	: Not applicable

Freezing point	: Not available

Boiling point	: Not available

Flammability	: Non flammable,Not applicable

Explosive limits	: Not available

Lower explosion limit	: Not available

Upper explosion limit	: Not available

Flash point	: 78 °C

Auto-ignition temperature	: Not available

Decomposition temperature	: Not available

pH	: Not available

Viscosity, kinematic	: Not available

Solubility	: Not available

Partition coefficient n-octanol/water (Log Kow)	: Not available Vapour pressure	: Not available

Vapour pressure at 50°C	: Not available

Density	: Not available

Relative density	: ≈ 1.07

Relative vapour density at 20°C	: Not available

Particle characteristics	: Not applicable

9.2. Other information

## Information with regard to physical hazard classes

Based on the test methods set out in the UN Manual of Tests and Criteria and in Part A of Regulation (EC) No 440/2008, the product is not classified for any physical hazard class

Explosives: not applicable
Flammable gases: not applicable
Aerosols: not applicable
Oxidising gases: not applicable
Gases under pressure: not applicable
Flammable liquids: not applicable
Flammable solids: not applicable
Self-reactive substances and mixtures: not applicable
Pyrophoric liquids: not applicable
Pyrophoric solids: not applicable
Self-heating substances and mixtures: not applicable
Substances and mixtures which, in contact with water, emit flammable gases: not applicable
Oxidising liquids: not applicable
Oxidising solids: not applicable
Organic peroxides: not applicable
Corrosive to metals: not applicable
Desensitised explosives: not applicable

## Other safety characteristics

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

The product is non-reactive under normal conditions of use, storage and transport.

10.2. Chemical stability

Not established.

10.3. Possibility of hazardous reactions

Not established.

10.4. Conditions to avoid

Direct sunlight. Extremely high or low temperatures.

10.5. Incompatible materials

Strong acids. Strong bases.

10.6. Hazardous decomposition products

fume. Carbon monoxide. Carbon dioxide.

SECTION 11: Toxicological information

11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

Acute toxicity (oral)	: Harmful if swallowed.

Acute toxicity (dermal)	: Harmful in contact with skin.

Acute toxicity (inhalation)	: Harmful if inhaled.

| CRANBERRY ALLSPICE #EU26363F |  |
| --- | --- |
| ATE CLP (oral) | 934.367 mg/kg bodyweight |
| ATE CLP (dermal) | 1931.021 mg/kg bodyweight |
| ATE CLP (dust,mist) | 4.931 mg/l/4h |
| Cinnamic aldehyde (104-55-2) |  |
| LD50 oral rat | 2220 mg/kg (Source: NLM_CIP) |
| LD50 oral | 2200 mg/kg bodyweight |
| LD50 dermal rabbit | 1260 mg/kg (Source: EPA_HPV) |
| LD50 dermal | 1100 mg/kg bodyweight |
| Eugenol (97-53-0) |  |
| LD50 oral rat | 1930 mg/kg (Source: NZ_CCID) |
| LD50 oral | 2500 mg/kg bodyweight |
| d-Limonene (5989-27-5) |  |
| LD50 oral rat | 4400 mg/kg (Source: CHEMVIEW) |
| d-Limonene (5989-27-5) |  |
| LD50 dermal rabbit | > 5 g/kg (Source: CHEMVIEW) |
| COUMARIN (91-64-5) |  |
| LD50 oral rat | > 5000 mg/kg (Source: JAPAN_GHS) |
| LD50 dermal rat | 293 mg/kg (Source: ECHA_API) |
| Methyl ester of rosin (partially hydrogenated) (8050-15-5) |  |
| LD50 oral rat | > 2000 mg/kg (Source: CHEMVIEW) |
| LD50 dermal rabbit | > 10000 mg/kg (Source: ECHA_API) |
| Bis(2-ethylhexyl) adipate (103-23-1) |  |
| LD50 oral rat | 5600 mg/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | 8410 mg/kg (Source: NLM_CIP) |
| LC50 Inhalation - Rat | > 5.7 mg/l/4h |
| Phenylpropyl alcohol (122-97-4) |  |
| LD50 oral rat | 2250 mg/kg (Source: NICNAS) |
| LD50 oral | 2275 mg/kg bodyweight |
| LD50 dermal rabbit | < 5000 mg/kg (Source: NICNAS) |
| LD50 dermal | 5000 mg/kg bodyweight |
| Nutmeg Oil (8008-45-5) |  |
| LD50 oral rat | 2620 mg/kg (Source: NLM_CIP) |
| LD50 dermal rabbit | > 10 g/kg (Source: NLM_HSDB) |
| Amyl salicylate (2050-08-0) |  |
| LD50 oral rat | 4100 mg/kg (Source: NZ_CCID) |
| LD50 oral | 2000 mg/kg bodyweight |
| LD50 dermal rabbit | > 5000 mg/kg (Source: CHEMVIEW) |

Skin corrosion/irritation	: Causes skin irritation.

Serious eye damage/irritation	: Causes serious eye irritation.

Respiratory or skin sensitisation	: May cause an allergic skin reaction.

Germ cell mutagenicity	: Not classified

Additional information	: Based on available data, the classification criteria are not met

Carcinogenicity	: May cause cancer.

Additional information	: Based on available data, the classification criteria are not met

| Eugenol (97-53-0) |  |
| --- | --- |
| IARC group | 3 - Not classifiable |
| d-Limonene (5989-27-5) |  |
| IARC group | 3 - Not classifiable |
| COUMARIN (91-64-5) |  |
| IARC group | 3 - Not classifiable |
| Bis(2-ethylhexyl) adipate (103-23-1) |  |
| IARC group | 3 - Not classifiable |

Reproductive toxicity	: Not classified

Additional information	: Based on available data, the classification criteria are not met

STOT-single exposure	: Not classified

Additional information	: Based on available data, the classification criteria are not met

STOT-repeated exposure	: Not classified

Additional information	: Based on available data, the classification criteria are not met

Aspiration hazard	: May be fatal if swallowed and enters airways.

Additional information	: Based on available data, the classification criteria are not met

# Information on other hazards

## Endocrine disrupting properties

## The mixture does not contain substances identified as having endocrine disrupting properties in accordance with the criteria set out in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0.1%.

## Other information

The mixture contains fragrance substances which may cause an allergic skin reaction in sensitised individuals. Vapours, mist or spray may cause eye and respiratory tract irritation. Harmful in contact with skin. May be fatal if swallowed and enters airways. Contains substance(s) with suspected carcinogenic effects; avoid exposure and follow the precautions given in this safety data sheet.

:

SECTION 12: Ecological information

# Toxicity

Ecology - general	: Toxic to aquatic life with long lasting effects. Harmful to aquatic life with long lasting effects.

Ecology - water	: Toxic to aquatic life with long lasting effects.

Hazardous to the aquatic environment, short–term (acute)

Hazardous to the aquatic environment, long–term (chronic)

: Not classified

: Harmful to aquatic life with long lasting effects.

| Eugenol (97-53-0) |  |
| --- | --- |
| LC50 - Fish [1] | 13 mg/l (Exposure time: 96 h - Species: Danio rerio [semi-static] Source: ECHA) |
| d-Limonene (5989-27-5) |  |
| LC50 - Fish [1] | 0.619 – 0.796 mg/l (Exposure time: 96 h - Species: Pimephales promelas [flow-through] Source: EPA) |
| LC50 - Fish [2] | 35 mg/l (Exposure time: 96 h - Species: Oncorhynchus mykiss Source: EPA) |
| Bis(2-ethylhexyl) adipate (103-23-1) |  |
| LC50 - Fish [1] | 0.48 – 0.85 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus [static] Source: EPA) |
| LC50 - Fish [2] | 0.48 – 0.85 mg/l (Exposure time: 96 h - Species: Oncorhynchus mykiss [static] Source: EPA) |
| EC50 - Crustacea [1] | > 1.6 mg/l (Exposure time: 48 h - Species: Daphnia magna) |
| EC50 72h - Algae [1] | > 500 mg/l (Species: Desmodesmus subspicatus) |

# Persistence and degradability

| CRANBERRY ALLSPICE #EU26363F |  |
| --- | --- |
| Persistence and degradability | May cause long-term adverse effects in the environment. Not established. |

  - Bioaccumulative potential

| CRANBERRY ALLSPICE #EU26363F |  |
| --- | --- |
| Bioaccumulative potential | Not established. |
| Cinnamic aldehyde (104-55-2) |  |
| Partition coefficient n-octanol/water (Log Pow) | 2.1065 (at 25 °C) |
| Eugenol (97-53-0) |  |
| Partition coefficient n-octanol/water (Log Pow) | 1.83 (at 30 °C (at pH 5.5) |
| d-Limonene (5989-27-5) |  |
| Partition coefficient n-octanol/water (Log Pow) | 4.38 (at 37 °C (at pH 7.2) |
| Methyl ester of rosin (partially hydrogenated) (8050-15-5) |  |
| Partition coefficient n-octanol/water (Log Pow) | 6.4 – 7.6 (at pH 6) |
| Bis(2-ethylhexyl) adipate (103-23-1) |  |
| BCF - Fish [1] | (27 dimensionless) |
| Partition coefficient n-octanol/water (Log Pow) | 8.94 (at 25 °C) |
| Phenylpropyl alcohol (122-97-4) |  |
| Partition coefficient n-octanol/water (Log Pow) | 1.6 (at 35 °C (at pH 7) |
| Amyl salicylate (2050-08-0) |  |
| BCF - Fish [1] | (1170 dimensionless (whole body w.w.) |
| Partition coefficient n-octanol/water (Log Pow) | 4.5 (at 30 °C) |

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

The mixture does not contain substances meeting the PBT or vPvB criteria in accordance with Annex XIII of Regulation (EC) No. 1907/2006 at a concentration equal to or greater than 0.1%.

12.6. Endocrine disrupting properties

The mixture does not contain substances identified as having endocrine disrupting properties in accordance with the criteria set out in Regulation (EC) No. 1907/2006, Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0.1%.

12.7. Other adverse effects

The mixture contains fragrance substances hazardous to the aquatic environment. Harmful to aquatic life with long lasting effects. Avoid release to the environment. No other adverse effects are known based on available information.

SECTION 13: Disposal considerations

13.1. Waste treatment methods

Waste treatment methods	: Dispose of contents/container in accordance with licensed collector’s sorting instructions.

Product/Packaging disposal recommendations	: Dispose in a safe manner in accordance with local/national regulations. Dispose of

contents/container in accordance with local/national laws and regulations.

Ecology - waste materials	: Avoid release to the environment.

HP Code	: HP5 - “Specific Target Organ Toxicity (STOT)/Aspiration Toxicity:” waste which can cause specific target organ toxicity either from a single or repeated exposure, or which cause acute toxic effects following aspiration.

HP6 - “Acute Toxicity:” waste which can cause acute toxic effects following oral or dermal administration, or inhalation exposure.

HP7 - “Carcinogenic:” waste which induces cancer or increases its incidence

HP4 - “Irritant – skin irritation and eye damage:” waste which on application can cause skin irritation or damage to the eye.

HP13 - “Sensitising:” waste which contains one or more substances known to cause sensitising effects to the skin or the respiratory organs.

HP14 - “Ecotoxic:” waste which presents or may present immediate or delayed risks for one or more sectors of the environment

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number or ID number |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.2. UN proper shipping name |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.3. Transport hazard class(es) |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.4. Packing group |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| 14.5. Environmental hazards |  |  |  |  |
| Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
| No supplementary information available |  |  |  |  |

# 14.6. Special precautions for user

## Overland transport

Not applicable

## Transport by sea

Not applicable

## Air transport

Not applicable

## Inland waterway transport

Not applicable

## Rail transport

Not applicable

# 14.7. Maritime transport in bulk according to IMO instruments

Not applicable

SECTION 15: Regulatory information

# Safety, health and environmental regulations/legislation specific for the substance or mixture

    - EU-Regulations

REACH Annex XVII (Restriction List)

| EU restriction list (REACH Annex XVII) |  |  |
| --- | --- | --- |
| Reference code | Applicable on | Entry title or description |
| 3(a) | d-Limonene ; Nutmeg Oil | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 2.1 to 2.4, 2.6 and 2.7, 2.8 types A and B, 2.9, 2.10, 2.12, 2.13 categories 1 and 2, 2.14 categories 1 and 2, 2.15 types A to F |
| 3(b) | CRANBERRY ALLSPICE #EU26363F ; Cinnamic aldehyde ; Eugenol ; d-Limonene ; Phenylpropyl alcohol ; Nutmeg Oil ; Amyl salicylate | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10 |
| 3(c) | CRANBERRY ALLSPICE #EU26363F ; Cinnamic aldehyde ; d-Limonene ; Methyl ester of rosin (partially hydrogenated) ; Nutmeg Oil ; Amyl salicylate | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1 |
| 40. | d-Limonene ; Nutmeg Oil | Substances classified as flammable gases category 1 or 2, flammable liquids categories 1, 2 or 3, flammable solids category 1 or 2, substances and mixtures which, in contact with water, emit flammable gases, category 1, 2 or 3, pyrophoric liquids category 1 or pyrophoric solids category 1, regardless of whether they appear in Part 3 of Annex VI to Regulation (EC) No 1272/2008 or not. |

REACH Annex XIV (Authorisation List)

Contains no substance(s) listed on REACH Annex XIV (Authorisation List)

## REACH Candidate List (SVHC)

Contains no substance(s) listed on the REACH Candidate List

## PIC Regulation (Prior Informed Consent)

Contains no substance(s) listed on the PIC list (Regulation EU 649/2012 concerning the export and import of hazardous chemicals)

## POP Regulation (Persistent Organic Pollutants)

Contains no substance(s) listed on the POP list (Regulation EU 2019/1021 on persistent organic pollutants)

## Ozone Regulation (1005/2009)

Contains no substance(s) listed on the Ozone Depletion list (Regulation EU 1005/2009 on substances that deplete the ozone layer)

## Explosives Precursors Regulation (2019/1148)

Contains no substance(s) listed on the Explosives Precursors list (Regulation EU 2019/1148 on the marketing and use of explosives precursors)

## Drug Precursors Regulation (273/2004)

Contains no substance(s) listed on the Drug Precursors list (Regulation EC 273/2004 on the manufacture and the placing on market of certain substances used in the illicit manufacture of narcotic drugs and psychotropic substances)

## National regulations

Germany

Employment restrictions	: Observe restrictions according Act on the Protection of Working Mothers (MuSchG). Observe restrictions according Act on the Protection of Young People in Employment (JArbSchG).

Water hazard class (WGK)	: WGK 3, Highly hazardous to water (Classification according to AwSV, Annex 1). List of sensitizing substances (TRGS 907)	: Contains sensitizing substances according TRGS 907.

Chemicals Prohibition Ordinance (ChemVerbotsV)	: This product is subject to ChemVerbotsV Annex 2 Entry 1. The following requirements must

be observed: authorization requirement (according to § 6 paragraph 1 sentence 1), basic requirements for carrying out the delivery (according to § 8 paragraph 1, 3 and 4), identification and documentation (according to § 9 paragraph 1 to 3) and exclusion of the shipping route (according to § 10).

Hazardous Incident Ordinance (12. BImSchV)	:  Is not subject of the Hazardous Incident Ordinance (12. BImSchV)

## Netherlands

ABM category	: A(2) - toxic for aquatic organisms, may have longterm hazardous effects in aquatic environment

SZW-lijst van kankerverwekkende stoffen	: Resin acids and Rosin acids, hydrogenated, methyl esters is listed SZW-lijst van mutagene stoffen	: Resin acids and Rosin acids, hydrogenated, methyl esters is listed SZW-lijst van reprotoxische stoffen – Borstvoeding  : None of the components are listed

SZW-lijst van reprotoxische stoffen – Vruchtbaarheid

: None of the components are listed

SZW-lijst van reprotoxische stoffen – Ontwikkeling	: None of the components are listed

## Denmark

Class for fire hazard	: Class III-1

Store unit	: 50 liter

Classification remarks	: Flammable according to the Danish Ministry of Justice; Emergency management guidelines for the storage of flammable liquids must be followed

Danish National Regulations	: Young people below the age of 18 years are not allowed to use the product

Pregnant/breastfeeding women working with the product must not be in direct contact with the product

The requirements from the Danish Working Environment Authorities regarding work with carcinogens must be followed during use and disposal

# Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

Other information	: None.

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Acute Tox. 3 (Dermal) | Acute toxicity (dermal), Category 3 |
| Acute Tox. 3 (Inhalation) | Acute toxicity (inhal.), Category 3 |
| Acute Tox. 3 (Oral) | Acute toxicity (oral), Category 3 |
| Acute Tox. 4 (Dermal) | Acute toxicity (dermal), Category 4 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment – Acute Hazard, Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment – Chronic Hazard, Category 1 |
| Aquatic Chronic 2 | Hazardous to the aquatic environment – Chronic Hazard, Category 2 |

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Aquatic Chronic 3 | Hazardous to the aquatic environment – Chronic Hazard, Category 3 |
| Asp. Tox. 1 | Aspiration hazard, Category 1 |
| Carc. 1B | Carcinogenicity, Category 1B |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Flam. Liq. 3 | Flammable liquids, Category 3 |
| H226 | Flammable liquid and vapour. |
| H301 | Toxic if swallowed. |
| H302 | Harmful if swallowed. |
| H304 | May be fatal if swallowed and enters airways. |
| H311 | Toxic in contact with skin. |
| H312 | Harmful in contact with skin. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H319 | Causes serious eye irritation. |
| H331 | Toxic if inhaled. |
| H332 | Harmful if inhaled. |
| H341 | Suspected of causing genetic defects. |
| H350 | May cause cancer. |
| H361 | Suspected of damaging fertility or the unborn child. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |
| H411 | Toxic to aquatic life with long lasting effects. |
| H412 | Harmful to aquatic life with long lasting effects. |
| Muta. 2 | Germ cell mutagenicity, Category 2 |
| Repr. 2 | Reproductive toxicity, Category 2 |
| Skin Irrit. 2 | Skin corrosion/irritation, Category 2 |
| Skin Sens. 1 | Skin sensitisation, Category 1 |
| Skin Sens. 1B | Skin sensitisation, category 1B |

Abbreviations and acronyms:

ACGIH – American Conference of Governmental Industrial Hygienists
ADR/RID – European Agreement concerning the International Carriage of Dangerous Goods by Road / Regulations concerning the International Carriage of Dangerous Goods by Rail
PPE – Personal protective equipment
CAS – Chemical Abstracts Service
CLP – Regulation on classification, labelling and packaging of substances and mixtures; Regulation (EC) No. 1272/2008
DNEL – Derived no-effect level
EC50 – Half maximal effective concentration
ECHA – European Chemicals Agency
EINECS – European Inventory of Existing Commercial Chemical Substances
EWC – European Waste Catalogue
ERC – Environmental release category
IARC – International Agency for Research on Cancer
IATA – International Air Transport Association
IMDG – International Maritime Dangerous Goods Code
LTEL – Long-term exposure limit
LC50 – Median lethal concentration
LD50 – Median lethal dose
MS – Member States
MWC – Maximum workplace concentration
NTP – National Toxicology Program
N/E – Not established
OELV – Occupational exposure limit value
OSHA – Occupational Safety and Health Administration
PBT – Persistent, bioaccumulative and toxic
PNEC – Predicted no-effect concentration
PROC – Process category
PC – Product category
RE – Repeated exposure
REACH – Regulation on Registration, Evaluation, Authorisation and Restriction of Chemicals
SCOEL – Scientific Committee on Occupational Exposure Limits
SDS – Safety data sheet
SE – Single exposure
STP – Sewage treatment plant
SU – Sector of use
STOT – Specific target organ toxicity
TLV-TWA – Threshold limit value – time-weighted average
STEL – Short-term exposure limit
VLE – Exposure limit value
vPvB – Very persistent and very bioaccumulative

The classification complies with	: ATP 12 Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.