SECTION 1: IDENTIFICATION OF THE SUBSTANCE/MIXTURE AND OF THE COMPANY/UNDERTAKING

  - Product identifier

Product name: OAKMOSS & AMBER 10773/2

Product class: fragrances

Product No.: 3302909000

UFI: C520-C02U-U000-12SE

  - Relevant identified uses of the substance or mixture and uses advised against

Relevant identified uses:

Perfumes, aromatic substances. Not intended for personal use in this form or concentration. The use descriptor system is provided in Section 16.2.

Uses advised against:

Use other than for the uses and purposes described above is not recommended.

  - Details of the supplier of the safety data sheet:

UAB “MANFIRA”
Company code: 302812448
Eišiškių pl. 127, LT-02184 Vilnius, Lithuania
Tel.: +370 633 11451
Email: info@craftshop.lt 
Website: www.craftshop.lt

Additional contacts:
Email: info@krevo.eu 
Website: www.krevo.eu

Email address of the person responsible for the safety data sheet: info@uficode.eu

Website address: www.uficode.eu

1.4. Emergency telephone number

Emergency telephone number: 112 (available 24 hours throughout the EU). For toxicological advice, contact the national poison centre in the country where the product is placed on the market. National contact details are available at: https://poisoncentres.echa.europa.eu/appointed-bodies

SECTION 2: HAZARDS IDENTIFICATION

| Hazard class | Hazard category | Hazard statement |
| --- | --- | --- |
| Skin sensitisation | 1 | H317 |
| Skin irritation | 2 | H315 |
| Eye irritation | 2 | H319 |
| Hazardous to the aquatic environment — chronic hazard | 1 | H410 |

2.2. Label elements

Labelling according to Regulation (EC) No 1272/2008:

| Hazard pictogram(s): |  |  |
| --- | --- | --- |
| Signal word |  | WARNING |
| Hazard statement(s): | H317 | May cause an allergic skin reaction. |
|  | H315 | Causes skin irritation. |
|  | H319 | Causes serious eye irritation. |
|  | H410 | Very toxic to aquatic life with long lasting effects. |

Precautionary statements:

P101 If medical advice is needed, have product container or label at hand.

P103 Read carefully and follow all instructions.

P261 Avoid breathing vapours.

P264 Wash hands thoroughly after handling.

P272 Contaminated work clothing should not be allowed out of the workplace.

P273 Avoid release to the environment.

P280 Wear protective gloves/protective clothing/eye protection/face protection.

P301+P312 IF SWALLOWED: Call a POISON CENTER/doctor if you feel unwell.

P330 Rinse mouth.

P302+P352 IF ON SKIN: Wash with plenty of water.

P333+P313 If skin irritation or rash occurs: Get medical advice/attention.

P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P337+P313 If eye irritation persists: Get medical advice/attention.

P362+P364 Take off contaminated clothing and wash it before reuse.

P391 Collect spillage.

P501 Dispose of contents/container in accordance with applicable regulations.

Hazardous components for labelling:

1-(1,2,3,4,5,6,7,8-octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one, linalool, Bergamot, ext., 3-methyl-4-(2,6,6- trimethyl-2-cyclohexen-1-yl)-3-buten-2-one.

2.3. Other hazards

PBT and vPvB: not applicable. Neither the mixture nor its components meet the PBT and/or vPvB criteria set out in Annex XIII to REACH.

Endocrine-disrupting properties: not applicable. The product does not contain any substances included in the list established in accordance with Article 59(1) as having endocrine-disrupting properties at a concentration equal to or greater than 0.1 % by weight.

The product does not contain substances identified as having endocrine-disrupting properties in accordance with the criteria laid down in Commission Delegated Regulation (EU) 2017/2100 or Commission Regulation (EU) 2018/605 at a concentration equal to or greater than 0.1 % by weight.

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

3.2. Mixtures

The product is a chemical mixture:

| Substance name | Identification numbers | % | Classification according to Regulation (EC) No 1272/2008 |
| --- | --- | --- | --- |
| 1-(1,2,3,4,5,6,7,8-octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one | CAS-No.: 54464-57-2 EC-No.: 259-174-3 | 1.239024 – 19.899024 | Skin Irrit. 2, H315 Skin Sens. 1, H317 Aquatic Chronic 1, H410 (M=1) |
| linalool | CAS-No.: 78-70-6 EC-No.: 201-134-4 IDX-No.: 603-235-00-2 REACH-no: 01-2119474016-42 | 11.818 | Skin Irrit. 2, H315 Skin Sens. 1B, H317 Eye Irrit. 2, H319 |
| 1,3,4,6,7,8-hexahydro-4,6,6,7,8,8-hexamethylindeno[5,6-c]pyran | CAS-No.: 1222-05-5 EC-No.: 214-946-9 IDX-No.: 603-212-00-7 REACH-no: 01-2119488227-29 | 2.478048 – 5.588048 | Aquatic Acute 1, H400 (M=1) Aquatic Chronic 1, H410 (M=1) |
| 2-tert-butylcyclohexyl acetate | CAS-No.: 88-41-5 EC-No.: 201-828-7 | 3.11 | Aquatic Chronic 2, H411 |
| [3R-(3α,3aβ,7β,8aα)]-1-(2,3,4,7,8,8a-hexahydro-3,6,8,8-tetramethyl-1H-3a,7-methanoazulen-5-yl)ethan-1-one | CAS-No.: 32388-55-9 EC-No.: 251-020-3 REACH-no: 01-2119969651-28 | 2.17 | Skin Sens. 1B, H317 Aquatic Acute 1, H400 (M=1) Aquatic Chronic 1, H410 (M=1) |
| Bergamot, ext. | CAS-No.: 89957-91-5 EC-No.: 289-612-9 REACH-no: 01-2120117613-65 | 1.86 | Flam. Liq. 3, H226 Asp. Tox. 1, H304 Skin Irrit. 2, H315 Skin Sens. 1, H317 Eye Irrit. 2, H319 Aquatic Chronic 3, H412 |
| 2,6-dimethyloct-7-en-2-ol | CAS-No.: 18479-58-8 EC-No.: 242-362-4 REACH-no: 01-2119457274-37 | 1.86 | Eye Irrit. 2, H319 |
| 3-methyl-4-(2,6,6-trimethyl-2-cyclohexen-1-yl)-3-buten-2-one | CAS-No.: 127-51-5 EC-No.: 204-846-3 REACH-no: 01-2120138569-45 | 1.239024 | Skin Sens. 1B, H317 Aquatic Chronic 2, H411 |
| oxacycloheptadec-10-en-2-one | CAS-No.: 28645-51-4 EC-No.: 249-120-7 | 0.0242580000000 – 0.954258 | Aquatic Chronic 2, H411 |
| methyl 2,4-dihydroxy-3,6-dimethylbenzoate | CAS-No.: 4707-47-5 EC-No.: 225-193-0 REACH-no: 01-2120762759-36 | 0.93 | Skin Sens. 1B, H317 |
| 2,2,6-trimethyl-α-propylcyclohexanepropanol | CAS-No.: 70788-30-6 EC-No.: 274-892-7 | 0.93 | Skin Sens. 1B, H317 |
| 3-ethoxy-4-hydroxybenzaldehyde | CAS-No.: 121-32-4 EC-No.: 204-464-7 REACH-no: 01-2119958961-24 | 0.31 | Eye Irrit. 2, H319 |
| 2-ethyl-3-hydroxy-4-pyrone | CAS-No.: 4940-11-8 EC-No.: 225-582-5 REACH-no: 01-2120758795-36 | 0.15 | Acute Tox. 4 (Oral), H302 |
| Terpineol | CAS-No.: 8000-41-7 EC-No.: 232-268-1 REACH-no: 01-2119553062-49 | 0.14928 | Skin Irrit. 2, H315 Eye Irrit. 2, H319 |
| (R)-p-mentha-1,8-diene | CAS-No.: 5989-27-5 EC-No.: 227-813-5 IDX-No.: 601-096-00-2 REACH-no: 01-2119529223-47 | 0.1244 | Flam. Liq. 3, H226 Asp. Tox. 1, H304 Skin Irrit. 2, H315 Skin Sens. 1B, H317 Aquatic Acute 1, H400 (M=1) Aquatic Chronic 3, H412 |
| pin-2(3)-ene | CAS-No.: 80-56-8 EC-No.: 201-291-9 REACH-no: 01-2119519223-49 | 0.07464 | Flam. Liq. 3, H226 Asp. Tox. 1, H304 Skin Irrit. 2, H315 Skin Sens. 1, H317 |
| geraniol | CAS-No.: 106-24-1 EC-No.: 203-377-1 IDX-No.: 603-241-00-5 REACH-no: 01-2119552430-49 | 0.0622 | Skin Sens. 1, H317 |
| 3,7,11-trimethyldodeca-1,6,10-trien-3-ol, mixed isomers | CAS-No.: 7212-44-4 EC-No.: 230-597-5 REACH-no: 01-2119457636-29 | 0.04976 | Skin Sens. 1B, H317 Aquatic Acute 1, H400 (M=1) Aquatic Chronic 1, H410 (M=1) |
| (-)-pin-2(10)-ene | CAS-No.: 18172-67-3 EC-No.: 242-060-2 REACH-no: 01-2119519230-54 | 0.03732 | Flam. Liq. 3, H226 Asp. Tox. 1, H304 Skin Irrit. 2, H315 Skin Sens. 1B, H317 Aquatic Acute 1, H400 (M=1) Aquatic Chronic 1, H410 (M=1) |
| caryophyllene | CAS-No.: 87-44-5 EC-No.: 201-746-1 REACH-no: 01-2120745237-53 | 0.02488 | Asp. Tox. 1, H304 Skin Sens. 1B, H317 Aquatic Chronic 4, H413 |

The full text of the hazard statements (H-statements) is provided in Section 16.5.

SECTION 4: FIRST AID MEASURES

4.1. Description of first aid measures

General information

Remove contaminated clothing and wash/clean it with suitable washing/cleaning agents before reuse. First-aid personnel should also pay attention to their own safety.

After skin contact: Wash the affected skin with running water and suitable cleaning agents (soap, body wash, etc.). Do not use solvents and/or thinners for washing. May irritate the skin and cause an allergic skin reaction. If symptoms occur, seek medical advice from a dermatologist. Remove contaminated clothing and footwear and any other contaminated and/or dirty items; clean and/or wash them thoroughly with suitable cleaning/laundering agents before reuse.

After eye contact: Do not rub the eyes. Tilt the head, hold the eyelids wide open and immediately rinse thoroughly with water, including under the eyelids. Remove contact lenses if present and easy to do, then continue rinsing. Rinse for at least 15 minutes.

After inhalation: If feeling unwell, move the affected person to fresh air. Keep the person in a position that allows free and easy breathing. Remove clothing that may restrict breathing (scarves, neckties, etc.).

After ingestion: Rinse the mouth. Give a glass of water or milk to drink. Do not induce vomiting unless under medical supervision. If vomiting occurs, keep the person’s head lowered to prevent fluids from re-entering the mouth or throat. Contact a poison centre for urgent treatment advice. Provide information on the composition of the preparation/substance as stated in the supplier’s SDS or on the label.

4.2. Most important symptoms and effects, both acute and delayed

Eyes: possible redness, tearing and irritation.

Ingestion: may cause nausea, vomiting, abdominal pain, dizziness and general weakness.

Skin: irritation and possible sensitisation.

Inhalation: at high concentrations, general symptoms of malaise, dizziness or nausea may occur.

4.3. Indication of any immediate medical attention and special treatment needed: Treat symptomatically. Continuous monitoring is not required; prolonged/continued effects are possible.

SECTION 5: FIREFIGHTING MEASURES

5.1. Extinguishing media

Suitable extinguishing media: water, alcohol-resistant foam, carbon dioxide (CO2), dry powder, sand and water mist.

Unsuitable extinguishing media: strong water jet.

5.2. Special hazards arising from the substance or mixture: Avoid breathing vapours and prevent high vapour concentrations in air. Fire produces dense smoke. Exposure to combustion products may be harmful to health. Closed containers exposed to fire should be cooled with water. Do not allow firefighting water to enter drains or nearby surface waters.

5.3. Advice for firefighters

Firefighters must use appropriate protective equipment and a self-contained breathing apparatus (SCBA) with a full-face mask operated in positive-pressure mode. Firefighter clothing conforming to EN 469, including helmets, protective boots and gloves, will provide a basic level of protection in chemical incidents.

SECTION 6: ACCIDENTAL RELEASE MEASURES

6.1. Personal precautions, protective equipment and emergency procedures

6.1.1. For non-emergency personnel: Use the personal protective equipment described in Section 8 and comply with the safety requirements of Section 7 to prevent contamination of the skin, eyes and personal clothing.

6.1.2. For emergency responders: Stop work if the product is spilled. Stop the leak and evacuate persons not involved in the emergency response. Avoid skin contact, prevent contact with eyes and do not inhale. Use respiratory protective equipment, resistant protective clothing and tightly fitting safety goggles.

6.2. Environmental precautions

Do not allow the product to enter drains, surface water, groundwater or drainage systems. Avoid release to the environment. Prevent spreading over a large area. Collect the spilled product, for example with a universal absorbent.

6.3. Methods and material for containment and cleaning up

Avoid the formation of dust. Clean up spills immediately. Absorb the spilled quantity with dry earth, sand or another absorbent material, place it in a suitable container and dispose of it in accordance with national requirements (Section 13). If the product enters drains, surface water, groundwater or the terrestrial environment, or if a large quantity is released over a large area, notify the competent authorities.

6.4. Reference to other sections

Information on personal protective equipment is provided in Section 8.

Information on disposal is provided in Section 13.

SECTION 7: HANDLING AND STORAGE

### 7.1. Precautions for safe handling

7.1.1. Information on safe handling:

Follow the recommendations in Section 8.

Dispose of the product in accordance with Sections 6.3 and 13.

Fire prevention measures: Store in a cool, dry place and protect from heat/cold. In the event of fire, cool containers with water spray. Keep fire extinguishers readily accessible.

Measures to prevent aerosol and dust formation: Avoid the formation of high vapour concentrations in air. Use suitable personal protective equipment as specified in Section 8.

Environmental protection measures: No special measures apply. Do not allow the product to enter drains, surface water, groundwater, drainage systems, soil or the terrestrial environment.

7.1.2. Advice on general occupational hygiene: Do not eat, drink or smoke when using this product. Wash hands before breaks and after handling the product. Avoid contact with skin and eyes. Do not inhale or ingest.

7.2. Conditions for safe storage, including any incompatibilities

Requirements for storage rooms and containers: Store in tightly closed original containers in a dry, cool and well-ventilated area, out of the reach of children, away from food, drink and animal feed, and protected from direct sunlight. Absorbent material should be readily available in the storage area for use in the event of a spill. Do not reuse empty containers.

Incompatible materials: Avoid contact with other unpackaged chemical substances.

Further information on storage conditions: Store at 10–30 °C. The product may change colour during prolonged storage. Crystals may form when stored in cool conditions. If necessary, warm slightly to 40 °C before use and mix thoroughly.

7.3. Specific end use(s)

See Sections 1.2 and 16.2.

SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

When protective equipment (PPE) is used, additional measures must be implemented: the duration of work (exposure duration) should reflect the additional physiological strain on the worker caused by the PPE. Certain PPE may also reduce the worker’s ability to use tools and communicate. Therefore, the worker should be in suitable health, particularly with regard to medical conditions that may affect PPE use, and an adequate seal between the body and the PPE must be ensured, taking account of factors such as scars and facial hair.

Employers and self-employed persons are legally responsible for issuing and managing PPE and ensuring its correct use in the workplace. They should therefore define and document an appropriate PPE-use policy, including worker training.

8.1. Control parameters

Occupational exposure limits for hazardous components of the mixture, including long-term and short-term limit values in workplace air:

No data available.

8.2. Exposure controls

8.2.1. Appropriate engineering controls

Exposure: work duration is unrestricted (up to 480 minutes per shift, 5 shifts per week).

Arrange regular monitoring of workplace air quality. Ensure that eyewash facilities and safety showers are available near work areas.

8.2.2. General protective and hygiene measures:

Avoid contact with the product during work whenever possible. Personal protective equipment must be worn. Do not eat, drink or smoke in the workplace to prevent the product from contacting the skin, mouth or eyes. Wash hands with suitable agents, such as soap, before breaks and after work. At the end of work, remove contaminated/dirty clothing, footwear, goggles and other contaminated items and clean/wash them thoroughly with suitable cleaning/laundering agents before reuse.

Eye/face protection

Wear tightly fitting safety goggles and a face shield when handling the product (EN 166). Prevent contact with the eyes.

Skin protection

Wear footwear covering the entire foot, long-sleeved clothing and antistatic protective clothing (EN 1149), or at least cotton, non-static work clothing. Barrier creams may be used to protect the skin against prolonged contact with the product. Prevent the product from entering footwear. Select body protection according to the concentration and quantity of hazardous substances at the workplace.

Hand protection

Use chemical-resistant, impermeable gloves conforming to an approved standard for handling chemicals (EN 374). The glove material must be resistant to the product. Nitrile rubber is suitable. Glove thickness should be at least 0.11 mm. Observe the maximum wearing period, permeation time, diffusion and degradation of the glove material. For prolonged/continuous use, suitable material is butyl rubber (nitrile rubber), thickness 0.6–0.8 mm, breakthrough time > 480 min. Contaminated protective gloves should be replaced immediately.

Respiratory protection

If required, for example when aerosols are formed, and in case of insufficient ventilation or prolonged exposure, use a mask or half-mask with a filter protecting against organic gases, vapours or aerosols (P1 filter).

Protection against thermal hazards

Not applicable.

8.2.3. Environmental exposure controls

Air: Control of product particle emissions to ambient air must be carried out in accordance with the applicable general methodology for calculating particulate emissions and relevant legislation.

Water: Do not discharge the product into sewers, surface water, groundwater or drainage systems.

Soil and terrestrial environment: Prevent release to the environment. No specific parameters and/or measures have been established for controlling the product’s impact on the terrestrial environment.

SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

9.1. Information on basic physical and chemical properties

| Colour | Light yellow to brown |
| --- | --- |
| Physical state | Liquid |
| Odour | Characteristic, pleasant |
| Odour threshold | No data available |
| pH | No data available |
| Melting point/freezing point | No data available |
| Crystallisation temperature | No data available |
| Flash point | >=75 °C |
| Evaporation rate | No data available |
| Upper/lower flammability or explosive limits | No data available |
| Vapour pressure (20 °C) | No data available |
| Vapour density | No data available |
| Density | ~ 1 g/cm³ |
| Solubility in water | Low |
| Partition coefficient: n-octanol/water | No data available |
| Auto-ignition temperature | No data available |
| Decomposition temperature | No data available |
| Viscosity | No data available |
| Explosive properties | Not explosive |

9.2. Other information:

9.2.1. Information with regard to physical hazard classes: Based on the UN Manual of Tests and Criteria and the test methods laid down in Part A of Regulation (EC) No 440/2008, the product is not assigned to any physical hazard class.

Explosives: not applicable

Flammable gases: not applicable

Aerosols: not applicable

Oxidising gases: not applicable

Gases under pressure: not applicable

Flammable liquids: not applicable

Flammable solids: not applicable

Self-reactive substances and mixtures: not applicable

Pyrophoric liquids: not applicable

Pyrophoric solids: not applicable

Self-heating substances and mixtures: not applicable

Substances and mixtures which, in contact with water, emit flammable gases: not applicable

Oxidising liquids: not applicable

Oxidising solids: not applicable

Organic peroxides: not applicable

Corrosive to metals: not applicable

Desensitised explosives: not applicable

9.2.2. Other safety characteristics: not applicable.

SECTION 10: STABILITY AND REACTIVITY

10.1. Reactivity

Stable under the recommended conditions of use and storage.

10.2. Chemical stability

Stable under the recommended conditions of use and storage.

10.3. Possibility of hazardous reactions

No hazardous reactions are expected under the recommended conditions of use and storage; however, vapours may form a flammable/explosive mixture with air.

10.4. Conditions to avoid

For all combustible liquids, avoid temperatures above the flash point; the operating temperature should be at least 5 °C below the flash point. Do not heat closed containers. Avoid contact with oxidising substances.

10.5. Incompatible materials

Contact with other unpackaged materials is not permitted.

10.6. Hazardous decomposition products

Contact with water or storage under the recommended conditions for one year should not produce hazardous decomposition products.

SECTION 11: TOXICOLOGICAL INFORMATION

11.1. Information on hazard classes as defined in Regulation (EC) No 1272/2008

Acute toxicity: The product is not classified as acutely toxic according to the applicable criteria.

Acute toxicity of components:

2-Ethyl-3-hydroxy-4-pyrone, CAS No. 4940-11-8
LD50, oral, rat: 1150 mg/kg body weight
LD50, dermal, rabbit: > 5000 mg/kg body weight

1-(1,2,3,4,5,6,7,8-Octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one, CAS No. 54464-57-2
LD50, oral, rat: > 5000 mg/kg body weight
LD50, dermal, rat: > 5000 mg/kg body weight

Linalool, CAS No. 78-70-6
LD50, oral, rat: 2790 mg/kg body weight
LD50, dermal, rabbit: 5610 mg/kg body weight

Skin corrosion/irritation: The product is classified as irritating to skin and eye (Skin and Eye Irrit. 2, H315, H319).

Skin sensitisation: The product is classified as a skin sensitiser (Skin Sens. 1, H317).

Respiratory sensitisation: The product is not classified as a respiratory sensitiser.

Germ cell mutagenicity: No mutagenic effects have been identified.

Reproductive toxicity: No reproductive toxicity has been identified.

Carcinogenicity: The mixture is not classified as carcinogenic.

Developmental toxicity: No developmental toxicity has been identified.

Specific target organ toxicity – single exposure: No specific target organ toxicity has been identified.

Specific target organ toxicity – repeated exposure: No specific target organ toxicity has been identified.

Aspiration hazard: Based on available data, the classification criteria are not met.

11.2. Information on other hazards

11.2.1. Endocrine-disrupting properties:

No substances with endocrine-disrupting properties are present according to the criteria of Regulations (EC) No 1907/2006, (EU) 2017/2100 and (EU) 2018/605.

11.2.2. Other information:

Hazards associated with the possibility of ignition or explosion:

The product is non-flammable.

Human health hazards and possible consequences of exposure:

Causes skin an eye irritation. May cause an allergic reaction.

SECTION 12: ECOLOGICAL INFORMATION

12.1. Toxicity

Based on available data, the product is classified as hazardous to the aquatic environment (Aquatic Chronic 1, H410).

Toxicity of the components of the mixture:

1-(1,2,3,4,5,6,7,8-Octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one, CAS No. 54464-57-2
LC50, fish, 96 h: 1.3 mg/l
EC50, aquatic invertebrates, 48 h: 1.38 mg/l
ErC50, algae, 72 h: > 2.6 mg/l

1,3,4,6,7,8-Hexahydro-4,6,6,7,8,8-hexamethylindeno[5,6-c]pyran, CAS No. 1222-05-5
LC50, fish, 96 h: 0.452 mg/l
EC50, aquatic invertebrates, 48 h: 0.282 mg/l
ErC50, algae, 72 h: 0.72 mg/l

2-tert-Butylcyclohexyl acetate, CAS No. 88-41-5
LC50, fish, 96 h: 1.111 mg/l
EC50, aquatic invertebrates, 48 h: 1.671 mg/l
EC50, algae, 96 h: 0.549 mg/l

[3R-(3α,3aβ,7β,8aα)]-1-(2,3,4,7,8,8a-Hexahydro-3,6,8,8-tetramethyl-1H-3a,7-methanoazulen-5-yl)ethan-1-one, CAS No. 32388-55-9
LC50, fish, 96 h: 2.3 mg/l
EC50, aquatic invertebrates, 48 h: 0.86 mg/l
EC50, algae, 96 h: 4.3 mg/l

12.2. Persistence and degradability

No data available.

12.3. Bioaccumulative potential

No data available.

12.4. Mobility in soil

No data available.

12.5. Results of PBT and vPvB assessment

PBT: Not applicable. vPvB: Not applicable.

12.6. Endocrine-disrupting properties: This mixture contains no substances with endocrine-disrupting properties according to the assessment criteria set out in Regulations (EC) No 1907/2006, (EU) 2017/2100 and (EU) 2018/605.

12.7. Other adverse effects: The product is classified as hazardous to the aquatic environment and contains components hazardous to aquatic organisms. Avoid release to the environment.

SECTION 13: DISPOSAL CONSIDERATIONS

13.1. Waste treatment methods

Product:

Product waste must be managed as hazardous waste in accordance with national requirements and rules approved by local authorities. The hazardous properties of the waste must be assessed and appropriate safety measures, labelling and information must be provided. Waste generation should be avoided or minimised wherever possible. This product, solutions and any by-products must be disposed of in accordance with waste-disposal legislation. Dispose of surplus and non-recyclable products through a licensed waste-disposal contractor. Waste must not be discharged into drains.

Packaging:

Waste generation should be avoided or minimised wherever possible. Packaging waste should be recycled. Incineration or landfill disposal should only be considered when recycling is not feasible.

Note: Waste codes are assigned on the basis of general product uses and may not correspond to contaminants arising during actual use. To assign the appropriate waste code, waste producers must assess the actual process in which the waste was generated and its contaminants.

Contaminated packaging:

Completely empty the packaging and dispose of it in accordance with applicable legislation (Waste Management Rules, EWC).

Warning: Empty containers may retain hazardous product residues. Do not refill or clean containers without appropriate instructions. Protect containers from excessive pressure; do not cut, weld, solder, drill, grind or expose them to heat. Keep away from flames, sparks, static electricity and other ignition sources.

Waste classification:

HP4 Irritant – skin irritation

HP13 Sensitising.

HP14 Ecotoxic.

SECTION 14: TRANSPORT INFORMATION

The product is subject to dangerous-goods transport requirements and classification under IMDG, IATA and ADR/RID.

|  |  | ADR – road transport RID – rail transport | ADN – inland waterway transport IMDG – sea transport | IATA – air transport |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 14.1. | UN number | 3082 |  |  |  |
| 14.2. | UN proper shipping name | ## ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. (1-(1,2,3,4,5,6,7,8-octahydro-2,3,8,8-tetramethyl-2-naphthyl)ethan-1-one) |  |  |  |
| 14.3. | Transport hazard class(es) | 9 |  |  |
| 14.4 | Packing group | III |  |  |
| 14.5. | Labelling |  |  |  |

14.6. Maritime transport in bulk according to IMO instruments.

Not applicable.

14.7. Additional information:

In the event of accidental spillage or fire during transport, refer to the instructions in Sections 5, 6, 7 and 8 above.

EmS code: F-A, S-F.

SECTION 15: REGULATORY INFORMATION

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

Regulation (EC) No 1907/2006 of the European Parliament and of the Council of 18 December 2006 concerning the Registration, Evaluation, Authorisation and Restriction of Chemicals (REACH), establishing a European Chemicals Agency, amending Directive 1999/45/EC and repealing Council Regulation (EEC) No 793/93, Commission Regulation (EC) No 1488/94, Council Directive 76/769/EEC and Commission Directives 91/155/EEC, 93/67/EEC, 93/105/EC and 2000/21/EC (Official Journal of the European Union L 396, 30.12.2006; corrigendum L 136/3, 29.5.2007);

Regulation (EC) No 1272/2008 of the European Parliament and of the Council of 16 December 2008 on classification, labelling and packaging of substances and mixtures, amending and repealing Directives 67/548/EEC and 1999/45/EC and amending Regulation (EC) No 1907/2006 (Official Journal of the European Union L 353, 31.12.2008);

Lithuanian Hygiene Standard HN 23 “Occupational exposure limit values for chemical substances. General requirements for measurement and exposure assessment”;

Lithuanian Hygiene Standard HN 36 “Prohibited and restricted substances”;

Regulations on the protection of workers from risks related to chemical agents at work and on the provision of personal protective equipment;

Law on Waste Management of the Republic of Lithuania;

Law on Packaging and Packaging Waste Management of the Republic of Lithuania;

Waste Management Rules;

European Agreement concerning the International Carriage of Dangerous Goods by Road (ADR);

International Maritime Dangerous Goods Code (IMDG);

International Air Transport Association Dangerous Goods Regulations (IATA DGR);

European Waste Catalogue (EWC);

Guidance on information requirements and chemical safety assessment, Chapter R.12: Use description.

15.2. Chemical safety assessment

A chemical safety assessment has not been carried out pursuant to Article 14 of REACH.

SECTION 16: OTHER INFORMATION

16.1. Indication of changes

The information provided complies with Regulation (EC) No 1907/2006 (REACH), as amended by Regulation (EU) 2020/878.

Revised: 2026-07-28.

Version: 2

16.2. Identified uses, use description and categories

Use descriptor system:

Sector of use (SU):

| SU22 | Professional use: public domain (administration, education, entertainment, services, craftsmen) |
| --- | --- |

Chemical product category (PC):

| PC28 | Perfumes, fragrances |
| --- | --- |

16.3. Abbreviations and acronyms

ADR/RID European Agreement concerning the International Carriage of Dangerous Goods by Road / Regulations concerning the International Carriage of Dangerous Goods by Rail

PPE Personal protective equipment

CAS Chemical Abstracts Service

CLP Classification, Labelling and Packaging Regulation; Regulation (EC) No 1272/2008

DNEL Derived no-effect level

EC50 Median effective concentration

ECHA European Chemicals Agency

EINECS European Inventory of Existing Commercial Chemical Substances

EWC European Waste Catalogue

ERC Environmental release category

H&S Health and safety

IARC International Agency for Research on Cancer

IATA International Air Transport Association

IMDG International Maritime Dangerous Goods Code

OEL-TWA Long-term occupational exposure limit value

LC50 Median lethal concentration

MEASE Metals Estimation and Assessment of Substance Exposure

MS Member States

NTP National Toxicology Program

N/E Not established

OELV Occupational exposure limit value

OSHA Occupational Safety and Health Administration

PBT Persistent, bioaccumulative and toxic

PNEC Predicted no-effect concentration

PROC Process category

PC Chemical product category

RE Repeated exposure

REACH Registration, Evaluation, Authorisation and Restriction of Chemicals

SCOEL Scientific Committee on Occupational Exposure Limits

SDS Safety data sheet

SE Single exposure

STP Sewage treatment plant

SU Sector of use

STOT Specific target organ toxicity

TLV–TWA Threshold limit value – time-weighted average

OEL-STEL Short-term occupational exposure limit value

VLE–MP Occupational exposure limit – average value, mg/m³ air

vPvB Very persistent and very bioaccumulative

16.4. References

Data provided by the European Chemicals Bureau (ECB), the European Chemicals Agency (ECHA), the Swedish Chemicals Agency (KemI), the International Labour Organization (ILO) and the TOXNET databases.

16.5. Full text of all relevant hazard statements referred to in Section 3 in accordance with Regulation (EC) No 1272/2008 (CLP):

| Flammable liquids | H226 | Flammable liquid and vapour |
| --- | --- | --- |
| Acute toxicity | H302 | Harmful if swallowed |
| Aspiration hazard | H304 | May be fatal if swallowed and enters airways |
| Skin irritation | H315 | Causes skin irritation |
| Skin sensitisation | H317 | May cause an allergic skin reaction |
| Serious eye irritation | H319 | Causes serious eye irritation |
| Specific target organ toxicity – single exposure | H335 | May cause respiratory irritation |
| Hazardous to the aquatic environment, acute | H400 | Very toxic to aquatic life |
| Hazardous to the aquatic environment, long-term | H410 | Very toxic to aquatic life with long lasting effects |
| Hazardous to the aquatic environment, long-term | H411 | Toxic to aquatic life with long lasting effects |
| Hazardous to the aquatic environment, long-term | H412 | Harmful to aquatic life with long lasting effects |
| Long-term aquatic hazard | H413 | May cause long lasting harmful effects to aquatic life |

16.6. Disclaimer

The information is correct to the best of our knowledge at the date of preparation of this safety data sheet and is applicable when the product is used under the specified conditions and for the purpose indicated on the packaging or in the technical recommendation. This is not a specification sheet, and the data provided should not be regarded as technical specifications. The information in this safety data sheet has been obtained from sources that we consider reliable. However, the information is provided without any express or implied warranty as to its accuracy. Certain information and conclusions in this document are derived from sources other than direct test data on the substance/mixture itself. The conditions or methods of handling, storage, use and disposal of the product are beyond our control and may be unknown to us. For these and other reasons, we accept no responsibility and expressly disclaim liability for loss, damage or expense arising out of or in any way connected with the handling, storage, use or disposal of this product. If the product is used as a component in another product, the information in this safety data sheet may not apply.

End of safety data sheet